# -----------------------------------------------

# Ti fine temp sweep: 300 -> 900 K step 1 K

# 5000 equilibration + 1000 production steps

# Single combined CSV with final production values only

# LAMMPS 7 Feb 2024-compatible syntax (no spaced math tokens)

# -----------------------------------------------

clear

units           metal

dimension       3

boundary        p p p

atom_style      atomic

# ---------- Parameters (define first) ----------

variable Tstart equal 300

variable Tend   equal 900

variable Tstep  equal 1

# compute difference and number of temps (LAMMPS-safe, no spaced operators)

variable tempdiff equal (v_Tend-v_Tstart)

variable Ntemps  equal (v_tempdiff/v_Tstep)+1

variable steps_equil equal 8000

variable steps_prod  equal 15000

variable seed equal 12345

# ---------- Output file (single combined CSV) ----------

variable outfile string "Ti_fine_temp_sweep.csv"

shell echo "Seed,T_target(K),Temp(K),Press(bar),PE(eV),KE(eV),Etot(eV),Vol(A^3)" > ${outfile}

# ---------- System setup (one-time) ----------

# Replace this with your own read_data or lattice/create commands as needed

# (create a modest cell; you can replace with read_data if available)

lattice         fcc 4.05

region          box block 0 10 0 10 0 10 units lattice

create_box      1 box

create_atoms    1 box

mass            1 47.867

# ---------- Potential (ensure file exists in working dir) ----------

pair_style      eam/alloy

pair_coeff      * * Ti_Zhou04.eam.alloy Ti

neighbor        2.0 bin

neigh_modify    delay 10 every 1 check yes

# ---------- Thermo (minimal) ----------

thermo          1000

thermo_style    custom step time temp press pe ke etotal vol

# ---------- Main loop over temperatures ----------

variable t loop ${Ntemps}

label loop_temp

# compute current temperature (must use this spacing style)

variable T equal ${Tstart}+(${t}-1)*${Tstep}

print ">>> Running T = ${T} K"

# initialize velocities for this T

velocity all create ${T} ${seed} mom yes rot yes dist gaussian

# Equilibration (NPT)

fix eq all npt temp ${T} ${T} 0.1 iso 0 0 1.0

run ${steps_equil}

unfix eq

# Production (NPT)

fix prod all npt temp ${T} ${T} 0.1 iso 0 0 1.0

run ${steps_prod}

# Capture the *final* production values (variables reflect instantaneous values)

# Note: in 'metal' units pressure is in bar (1 bar = 1e5 Pa)

variable Tempvar equal temp

variable Pressvar equal press

variable PEvar equal pe

variable KEvar equal ke

variable Etotvar equal etotal
  variable Volvar equal vol

# Append a single line to the combined CSV with the final values

# Use 'print ... append <file>' to append to the CSV (no extra spaces in expression)

print "${seed},${T},${Tempvar},${Pressvar},${PEvar},${KEvar},${Etotvar},${Volvar}" append ${outfile}

unfix prod

next t

jump SELF loop_temp

print "✅ All done. Combined data saved to: ${outfile}"